Lecture 29

Consider now the following geometry ：

The points on the source  and  emit light incoherently with respect to each other if the source is a classical (thermal or fluorescent) source. (Only if the source is the output of a laser will  and  have a definite phase relationship. For a classical source, there is no way for the emitting dipoles at  and  to “talk to each other” and thereby establish any definite phase relationship.) So, what happens at P?

Both  and  contribute to the fringe at P, because the angle for constructive (or destructive) interference is the same.

- The rays from, taking paths A and B add coherently at P

- The rays from, taking paths C and D add coherently at P

- However,adds incoherently to  at P (each source point produces its own fringes in the observation plane, and the intensities of these add .

Michelson Interferometer

These are many interferometer which in essence result in the coherent addition of two waves (for a discussion of many of them, see Born+Wolf chap.7).The final one we will consider is the Michelson interferometer. This interferometer will be particularly useful for us in our consideration of coherence. The basic optical set up is as follows:

, are identical, R=1 reflectors.

The partially reflecting coating on the beam splitter is designed to reflect 50% of the light, so equal intensities propagate in each “arm” of the interferometer.

The compensation plate is included so that the light passing through each arm goes through the thickness of glass of the beam splitter exactly three times.

- Each beam also reflects off the beam splitter once and is transmitted once. Thus if the mirrors  and  have equal distances from the beam splitter, then the optical paths are identical for the two arms , so there must be constructive interference at P.

- A note on mirrors and phase shifts :

It can be shown from quite general considerations (power conservation and time reversal symmetry in particular), that for a lossless dielectric mirror, the reflected and transmitted waves have a phase shift with respect to each other. Here the wave is reflected from an effective single surface (or “reference plane”) with amplitude reflectivity r and transmissivity t, where  (r, t real )

Extensive discussion and proof of these points can be found in Siegman, Lasers, chap.11.1

Haus, Wave+ Fields in Optoelectrances chap.3.

Thus to calculate the reflected or transmitted fields in a Michelson Interferometer, consider the following diagram:

Reflected field:

If (50% beam splitter),  then  (no reflection!)

Transmitted field:

If , then r=t= and 2rt=1

So  and  (complete transmission!)

The consequence of this is simple. When the mirrors are equidistant from the beam splitter, the optical paths for light propagating from S to P in each arm are identical. However, there is a  phase shift between the waves from the two arms propagating back toward the source S. Thus there is complete destructive interference in the “reflected” direction and complete contructive interference in the “transmitted” direction,i.e. towards point P. Thus all the light from S (along the axis of the interferometer) ends up at P; more is reflected back towards the source.

Now consider the intensity at P as one of the mirrors is moved a distance d:

The optical path length in that arm changes by an amount 2d (assuming an index n=1 , as is usually the case). Thus the phase difference is

If the beam splitter is exactly 50% , =>

Where  is the retardation time, or optical delay caused by displacing the mirror by distance d.

Intensity pattern on observation screen:

Note that the point source S puts out spherical waves.

The easiest way to understand the intensity pattern is to realize that mirrors  and  give rise to virtual images of S as seen from the observation screen:

(see Moore pattern and 2 point sources displaced vertically )

The off-axis waves will also give rise to constructive interference when

m= integer

- Get fringes of equal inclination

- Rings appear when you look from P towards S

e.g. for  :

if the source emits several wavelength, the rings will appear colored, since different wavelengths satisfy the constructive interference condition at different angles. A nice picture is given in Guenther color plate 4-19.

Tyman-Green Interferometer

This is an important variation on the Michelson interferometer. The basic idea is to put a collimating lens in front of the source so that plane waves rather than spherical waves are incident on the interfetometer.

The value of the T-G interferometer is that a single fringe is observed when  and  are flat (i.e. the screen is uniformly bright or dark if the conditions for constructive on destructive interference are met).

Now, optical components may be placed in one arm of the interferometer in order to see their effect on the wavefront.

e.g. wedge (in what might normally be an optical flat )

e.g. test lens aberrations

- see Born+ Wolf figs.7.41 and 7.42

(+ compare of Born+ Wolf figs 5.31)